# --- Setup Variables ---

temp_val = 0

sound_val = 0

light_val = 0

moisture_val = 0

mode = 0

status = "Normal"

# CSV Header

serial.write_line("Temperature,Sound,Light,Moisture,Status")

def on_forever():

global temp_val, sound_val, light_val, moisture_val, status

# 1. Read Sensors

temp_val = input.temperature()

sound_val = input.sound_level()

moisture_val = pins.analog_read_pin(AnalogPin.P0)

# 2. Advanced Light Reading (Toggling screen for accuracy)

led.enable(False) # Turn off LEDs to sense light

light_val = input.light_level()

led.enable(True)  # Turn LEDs back on for display

# 3. Check Conditions (Based on your Flowchart)

# FIRE ALARM: Temp > 45 AND Moisture < 300

if temp_val > 45 and moisture_val < 300:

status = "FIRE_ALARM"

basic.show_icon(IconNames.ANGRY)

music.play_melody("C5 B C5 B C5 B C5 B", 120) # Siren effect

# LOW MOISTURE: Moisture < 200

elif moisture_val < 200:

status = "LOW_MOISTURE"

basic.show_icon(IconNames.SAD)

music.play_melody("C E G B A G F E ", 120) # Sad melody

# NOISY: Sound > 180

elif sound_val > 180:

status = "NOISY"

basic.show_icon(IconNames.CONFUSED)

music.play_tone(131, music.beat(BeatFraction.WHOLE)) # Low tone

# OTHERWISE: Normal

else:

status = "Normal"

basic.clear_screen()

# 4. Create and Send CSV String

data_row = str(temp_val) + "," + str(sound_val) + "," + str(light_val) + "," + str(moisture_val) + "," + status

serial.write_line(data_row)

basic.pause(2000)

basic.forever(on_forever)

# --- Button Logic for Testing ---

def on_button_pressed_a():

global mode

mode = (mode + 1) % 4

letters = ["T", "S", "L", "M"]

basic.show_string(letters[mode])

input.on_button_pressed(Button.A, on_button_pressed_a)

def on_button_pressed_b():

if mode == 0: basic.show_number(temp_val)

elif mode == 1: basic.show_number(sound_val)

elif mode == 2: basic.show_number(light_val)

elif mode == 3: basic.show_number(moisture_val)

input.on_button_pressed(Button.B, on_button_pressed_b)